### abdomen / pelvis w IV

application		abdominal pain, appendicitis, diverticulitis, pancreatitis, SBO, routine 			oncology indications, etc

landmarks		diaphragm through symphysis

Oral/IV contrast	CT Contrast Dosing Guidelines Oral & IV.pdf

rate			(see Oral/IV contrast document above)

delay			Vendor specific bolus timing software.  Otherwise delay ~ 60-65 sec

image order		axial:  cranial → caudal

coronal:  anterior → posterior

sagittal:  right → left

comments

submit 2.5 x 2.5 mm axial (no thicker than 3 mm).

submit 3 x 3 mm coronal & sagittal MPR

patient’s arms up. If unable to raise arms, please document reason

approved, body subcommittee 6-11-24