Synergy CT Enterography protocol

- Fasting 3 hours prior to procedure.

- IV: 18 or 20G antecubital fossa.

- Contrast: VoLumen or Breeza CT Enterography:

- Protocol 450 ml pre-mixed bottles
- Protocols involve the patient drinking either 900 ml or 1350 ml
- Oral contrast sequence is,

| 450 ml VoLumen / Breeza | 0-10 minutes |
| --- | --- |
| 450 ml VoLumen / Breeza | 10-20 minutes |
| 450 ml water | 20-30 minutes |

Do not exceed 60 min drinking to scanning time, as contrast will likely already be in the colon. Give last oral water dose just prior to scanning. (Note: Incidence of adverse reaction to Sorbitol increases with VoLumen ingestion over 1350mL).

IV Contrast:  100 mL Isovue 300 @ 4mL/sec

Meds: Glucagon 0.1 mg IV slowly over 3-5 min prior to scanning (Give glucagon only if requested; to be administered by Nurse).

Start injection and scan with 60 sec delay with 64-slice scanner or higher at 64x0.5 mm collimation (For 16-slice use 16x0.75mm). Use dose modulation, if available. Otherwise, 120kVp/0.4 sec gantry rotation and 180 effective mAs should be used. Contrast delay should not exceed 75 seconds.

- Reconstructions:

  - Axial dataset @ 2.5 mm thickness with 2.5 mm overlap.

  - Coronal dataset @ 3 mm thickness at 3 mm intervals encompassing entire bowel.

  - Send thin slice dataset to the workstation for 3D volume rendering.